BIG PICTURE

How we find the car position from a calibrated camera?

- Assume the camera is already calibrated (intrinsics K and distortion are known, and images are undistorted).

- Each new image gives pixel measurements.

- Geometry converts those pixels into ground coordinates.

- Then we estimate the car pose and compare with other sensors for redundancy.

Step-by-step loop (what we do at every time step i):

1) Acquire image I_i and undistort it using the calibration.

2) Detect reliable feature points on the ground (typically corners) in the undistorted image.

3) Convert each detected pixel to a ground-plane point (X,Y) using the ground-plane model (homography or ray–plane intersection).

4) Match current ground points to stored map landmarks (descriptor matching + geometric gating).

5) Reject false matches using RANSAC.

6) Estimate the car pose on the map (x, y, psi) by fitting the planar rigid-motion model to the inlier matches (least squares / SVD).

7) Update the vehicle position estimate and compute incremental travel distance from consecutive poses.

8) Redundancy: compare the vision-based pose with odometry/IMU (form residuals). If residuals are too large, flag a fault or down-weight the bad channel.

9) Repeat for the next frame.

qCarPosition2 pipeline

[Camera image I_i]

|  (undistort using calibration)

v

[Undistorted image]

|  (detect corners / features)

v

[Pixel points (u,v)]

|  (pixel -> ground using homography / ray-plane)

v

[Ground points (X,Y)]

|  (match to map landmarks)

v

[Candidate correspondences]

|  (RANSAC outlier rejection)

v

[Inlier correspondences]

|  (pose fit on ground: rotation + translation)

v

[Car pose on map: (x,y,psi)]

|  (difference between i and i-1)

v

[Distance traveled + position update]

|

+--> [Redundancy check vs odometry/IMU: residuals -> fault detection/fusion]

# 1. The problem we are solving

## 1.1 What we measure and what we want

We have a camera fixed to the vehicle. The camera sees the ground. From each image we want the vehicle pose on a ground map.

Pose on the ground map means: position (x, y) and heading (psi).

We want to build a map from the camera (during mapping).

Later, we want to localize the vehicle on that map (during operation).

## 1.2 Why redundancy matters

If you estimate position from only one source, a single failure can destroy the estimate (wheel slip, temporary glare, bad matches, etc.).

Mathematical redundancy means: compute position in two or more independent ways and compare them.

Camera-map position estimate (vision-based).

Wheel odometry position estimate (kinematics-based).

IMU yaw estimate (inertial-based).

Optional: GPS or other sensors (if available).

The comparisons produce residuals. Residuals close to zero mean consistency. Large residuals indicate a fault or a condition change.

# 2. Frames and what (x, y, psi) means

## 2.1 Frames used in qCarPosition projects

{M}: map frame (ground plane). This is the coordinate system of the map we create.

{C}: camera frame (fixed to the camera).

{I}: image frame (pixel coordinates).

{B}: body frame (vehicle). The vehicle pose is expressed on the map.

## 2.2 Vehicle pose on the map

On the ground, the vehicle pose is the translation plus yaw (heading):

t_B =
[ x ]
[ y ]

R(psi) =
[ cos(psi)  -sin(psi) ]
[ sin(psi)   cos(psi) ]

If you place a point in the body frame and want its coordinates in the map frame, you rotate and then translate. In these notes we use this same rotate-then-translate logic repeatedly.

# 3. The pinhole camera model (the basic 'camera equation')

## 3.1 The intrinsic matrix K

K converts a direction in the camera into pixel units.

K =
[ fx   0   cx ]
[  0  fy   cy ]
[  0   0    1 ]

fx, fy: focal lengths measured in pixels.

cx, cy: principal point (the pixel that corresponds to the optical axis).

## 3.2 Distortion (why undistort first)

Real lenses bend rays. The pinhole model assumes no bending. Therefore:

Calibrate distortion (radial/tangential).

Undistort each image before using K and any projective geometry.

In class: emphasize that undistortion is not a 'nice-to-have'. If you skip it, your homography and pose will be biased.

## 3.3 Homogeneous pixel vector

We represent a pixel (u, v) as a 3-vector:

p =
[ u ]
[ v ]
[ 1 ]

This is not a 3D point. It is a 2D point stored in a convenient projective form.

# 4. Camera mounting (extrinsics)

## 4.1 Rotation and translation from map to camera

Let a 3D point in the map frame be:

Xm =
[ X ]
[ Y ]
[ Z ]

The same point in the camera frame is:

Xc = R Xm + t

where:

R =
[ r11  r12  r13 ]
[ r21  r22  r23 ]
[ r31  r32  r33 ]

t =
[ t1 ]
[ t2 ]
[ t3 ]

R and t are determined by measuring the camera mount (height, pitch, roll, yaw) or by calibration procedures.

## 4.2 The ground plane assumption

In qCarPosition we assume the ground is a plane in the map frame:

Z = 0

So every ground point is:

Xm =
[ X ]
[ Y ]
[ 0 ]

This assumption is what allows pixel-to-ground conversion using a 3x3 homography.

# 5. Deriving the ground-plane homography (the key mathematics)

## 5.1 Build the 3x3 matrix A

When Z = 0, only the first two columns of R matter. Create A by taking those two columns and appending t:

A =
[ r11  r12  t1 ]
[ r21  r22  t2 ]
[ r31  r32  t3 ]

This A maps a 2D ground point (X, Y) (written in homogeneous form) into a 3D camera ray direction before applying K.

## 5.2 Homography H and its inverse G

Define:

H = K A

This H maps a ground point in homogeneous form to a pixel in homogeneous form.

Assume det(H) is not zero, then define:

G = H^{-1}

## 5.3 Ground point P and pixel point p

Ground (map plane) homogeneous point:

P =
[ X ]
[ Y ]
[ 1 ]

Pixel homogeneous point:

p =
[ u ]
[ v ]
[ 1 ]

## 5.4 Pixel-to-ground computation (explicit normalization)

This is the computation you do for every detected feature pixel:

Compute q = G p.

Write q as a 3-vector [q1, q2, q3]^T.

Divide the first two entries by the third entry.

q = G p =
[ q1 ]
[ q2 ]
[ q3 ]

X = q1 / q3
Y = q2 / q3

Important classroom point: the division by q3 is where noise can be amplified, especially for far-away points (small q3).

## 5.5 What can go wrong with the homography

If the point is not on the ground plane (for example a wall corner), the computed (X, Y) will be wrong.

If the camera pitch/height is wrong (bad extrinsics), all projected points will be systematically shifted.

If the image is not undistorted, the mapping will curve and the map will drift.

# 6. Building the map (mapping stage)

## 6.1 What the map contains

The map contains ground points (landmarks) stored in map coordinates. A landmark is a 2-vector:

m_i =
[ Xi ]
[ Yi ]

We store many of these points collected over time.

## 6.2 The mapping loop (what you do per image)

Undistort the image.

Detect corners (interest points).

For each corner pixel (u, v), build p and compute (X, Y) using q = G p and normalization.

Reject points that are inconsistent with the ground plane (gating).

Insert accepted ground points into the map (store as landmarks or integrate into a mosaic).

## 6.3 Ground-plane gating (keep only road points)

Example gating rules (conceptual):

Reject points whose projected (X, Y) lie outside the feasible road region.

Reject points whose normalization is unstable (very small q3).

Reject points that come from sky/vehicle hood areas (region-of-interest in the image).

# 7. Selecting corners (why corners, and how)

## 7.1 The core idea: image must change in two directions

A corner is a location where intensity changes strongly in two independent directions. We quantify this using the structure tensor.

## 7.2 What Delta I means

Pick a location (u0, v0). Move by (Du, Dv).

Delta I is the brightness change:

Delta I = I(u0 + Du, v0 + Dv) - I(u0, v0)

For small moves, we approximate:

Delta I is approximately Iu*Du + Iv*Dv, where Iu and Iv are image derivatives.

## 7.3 The 2x2 matrix M (structure tensor)

Over a small neighborhood, compute:

M =
[ sum(Iu^2)    sum(Iu*Iv) ]
[ sum(Iu*Iv)   sum(Iv^2)  ]

Interpretation you can say in class: M is a machine that takes a direction [Du, Dv]^T and predicts how much the image will change in that direction.

If M has two large eigenvalues, it is a corner. If only one is large, it is an edge. If both are small, it is flat.

## 7.4 Practical corner selection steps (board-friendly)

Compute a corner score at each pixel (from M).

Keep only points above a threshold (strong corners).

Apply non-maximum suppression (keep only local maxima).

Enforce a minimum spacing so points are not clustered.

Prefer points in the road region (ROI).

# 8. Seeing the same point again (matching)

## 8.1 Why pixel location is not enough

The same ground point will appear at different pixel coordinates when the car moves. So we do not match by (u, v).

## 8.2 Descriptors (the 'fingerprint' of a corner)

For each corner, we compute a descriptor: a compact representation of the small image patch around the corner.

A good descriptor is similar for the same physical point under moderate viewpoint/lighting changes.

Descriptors allow us to propose candidate matches between current corners and stored map corners.

## 8.3 Candidate match test

For each current corner, compare its descriptor to descriptors in the map. Keep the best candidates. This step creates many false matches, so geometry must confirm the correct ones.

# 9. Pose estimation from matches (localization stage)

## 9.1 What we have at localization time

From the current image: a set of corner pixels.

From each corner pixel: a ground point yk = [xk, yk]^T (computed using G and normalization).

From the map: stored landmarks mk = [Xk, Yk]^T.

From descriptors: candidate correspondences (yk <-> mk).

## 9.2 The rigid-motion model on the ground

We assume the relationship between the current ground points and the map landmarks is a planar rigid motion: rotate then translate.

R(psi) =
[ cos(psi)  -sin(psi) ]
[ sin(psi)   cos(psi) ]

t =
[ x ]
[ y ]

Model for each matched pair:

mk = R(psi) yk + t + ek

## 9.3 Why at least 2 points are needed for yaw (intuition)

One point can be translated to match another point in infinitely many ways; rotation is not determined. Two non-identical points determine both translation and rotation (unless they collapse into the same location).

In practice we use many points to be robust.

# 10. RANSAC (why we must reject false matches)

## 10.1 The problem

Descriptor matches produce false correspondences. If you include them in least squares, the pose estimate becomes wrong.

## 10.2 The RANSAC idea (simple statement)

Randomly pick a small number of correspondences.

Compute a pose hypothesis from them.

Apply that pose to all current points and measure errors to their matched map points.

Count how many correspondences have small error (inliers).

Repeat; keep the pose with the most inliers.

Refine the pose using least squares on the inliers only.

## 10.3 What is 'small error'

After applying a pose (R, t), the residual for a pair is:

distance = length of mk - (R yk + t)

If distance < epsilon, the pair is an inlier.

# 11. Least squares solved with SVD (detailed Procrustes)

## 11.1 The least squares problem (in words)

Among all rotations R and translations t, choose the ones that make the transformed current points as close as possible to the matched map points.

## 11.2 Step A: compute the means (centroids)

Compute the mean of current points:

y_mean =
[ x_mean ]
[ y_mean ]

x_mean = (1/m) * sum(xk)
y_mean = (1/m) * sum(yk)

Compute the mean of map points:

m_mean =
[ X_mean ]
[ Y_mean ]

X_mean = (1/m) * sum(Xk)
Y_mean = (1/m) * sum(Yk)

## 11.3 Step B: center the points (remove translation)

Define centered points by subtracting the means:

y_centered(k) =
[ xk - x_mean ]
[ yk - y_mean ]

m_centered(k) =
[ Xk - X_mean ]
[ Yk - Y_mean ]

After centering, translation is removed. The remaining problem is to find the best rotation aligning centered sets.

## 11.4 Step C: build the 2x2 matrix S

For each k, form the 2x2 matrix outer product:

y_centered(k) * (m_centered(k))^T =

[ xk - x_mean ] [ Xk - X_mean   Yk - Y_mean ]
[ yk - y_mean ]

= [ (xk-x_mean)(Xk-X_mean)   (xk-x_mean)(Yk-Y_mean) ]
  [ (yk-y_mean)(Xk-X_mean)   (yk-y_mean)(Yk-Y_mean) ]

Now sum over k to get S:

S =
[ s11  s12 ]
[ s21  s22 ]

s11 = sum( (xk-x_mean)(Xk-X_mean) )
s12 = sum( (xk-x_mean)(Yk-Y_mean) )
s21 = sum( (yk-y_mean)(Xk-X_mean) )
s22 = sum( (yk-y_mean)(Yk-Y_mean) )

## 11.5 Step D: SVD of S

Compute the singular value decomposition:

S = U Sigma V^T

U =
[ u11  u12 ]
[ u21  u22 ]

Sigma =
[ sigma1   0     ]
[   0    sigma2  ]

V =
[ v11  v12 ]
[ v21  v22 ]

U and V are orthogonal (rotations/reflections). Sigma contains nonnegative singular values.

## 11.6 Step E: build the rotation R

Compute:

M = V U^T

If det(M) = +1, use R = M.

If det(M) = -1, correct it by flipping one axis:

D =
[ 1  0 ]
[ 0 -1 ]

R = V D U^T

## 11.7 Step F: recover translation t

Finally:

t = m_mean - R y_mean

t =
[ x ]
[ y ]

## 11.8 Heading angle psi from R

If

R =
[ r11  r12 ]
[ r21  r22 ]

then

psi = atan2(r21, r11)

# 12. Numerical toy example (2–3 points)

## 12.1 Example setup

Suppose the map contains points:

m1 = [ 2 ]    m2 = [ 4 ]    m3 = [ 2 ]
     [ 1 ]         [ 1 ]         [ 3 ]

Suppose the current measurements (from the camera and homography) are:

y1 = [ 0 ]    y2 = [ 2 ]    y3 = [ 0 ]
     [ 0 ]         [ 0 ]         [ 2 ]

Interpretation: the current set is a square corner at the origin, and the map set is the same shape shifted/rotated.

In class you can walk through means, centered points, S, SVD conceptually, and show how R and t appear.

## 12.2 How a wrong guess fails (visual idea)

If you apply a wrong yaw, the transformed points miss their intended map points. Distances become large. In RANSAC language: the number of inliers collapses.

A good classroom demonstration: draw the y-points as a small triangle, draw the m-points, then show correct rotation/translation aligns them; wrong rotation leaves a gap.

# 13. Redundancy (the heart of your lecture title)

## 13.1 Two independent position channels

Channel A: vision-based position from the camera-built map.

Channel B: kinematics-based position from wheel odometry (plus yaw from IMU).

## 13.2 Residuals (differences)

Define residuals between channels:

rx = x_cam - x_odo
ry = y_cam - y_odo
rpsi = psi_cam - psi_imu

Residuals close to zero mean the channels agree. Large residuals mean at least one channel is wrong or the environment changed (slip, glare, miscalibration, etc.).

## 13.3 Fault detection logic

If rx and ry jump suddenly: possible wheel slip or a bad vision match burst.

If rpsi grows slowly: possible calibration drift (camera pitch change) or IMU bias.

If camera reports few inliers: temporarily ignore camera and rely on odometry/IMU until camera becomes reliable again.

## 13.4 Fusion (high-level, classroom safe)

Use odometry/IMU as a prediction and camera-map as a correction. This reduces drift and increases robustness. The mathematical engine can be an extended Kalman filter, but the class message is: prediction + correction + residual monitoring.

# 15. At the end of the studies solve this problems

## 15.1 Compute one pixel-to-ground projection

Given K and A (numerical), compute H, G, then project one pixel p to obtain (X,Y).

## 15.2 Explain why edges are poor for pose

Edges constrain position strongly in one direction only; corners constrain in two directions.

## 15.3 RANSAC conceptual

Explain why many candidate matches are wrong and how inlier counting rejects them.

## 15.4 Redundancy

Give a scenario where odometry fails (slip) and camera succeeds, and the opposite (glare), and show residual behavior.

Same idea in 3 different versions

Version 1 — Plain-English narrative

At each time step, the camera acquires a new image.
Because the camera is already calibrated, the image is first undistorted.
From this corrected image we detect reliable visual features, typically corners on the ground.
Each detected pixel is then converted into a point on the ground plane using the ground-plane geometry.
These ground points are matched against known landmarks in the map.
Outliers are rejected using RANSAC, leaving only geometrically consistent matches.
From the remaining correspondences we estimate the car pose on the ground, namely its position and orientation.
By comparing the pose at consecutive time steps we compute the distance traveled and update the car position.
In parallel, the vision-based estimate is compared with odometry and IMU measurements to provide redundancy and fault detection.

Version 2 — Step-by-step loop

At each frame i:

- Acquire image Iiand undistort it using the camera calibration.

- Detect stable visual features in the image.

- Convert each pixel location into a ground-plane point using the homography (or ray–plane intersection).

- Match the current ground points with landmarks in the map.

- Reject incorrect matches using RANSAC.

- Estimate the car pose on the ground (position and heading) from the inlier correspondences.

- Compare the pose at time iwith the pose at time i-1to compute traveled distance and update position.

- Compare the vision-based pose with odometry and IMU to detect inconsistencies and provide redundancy.

Version 3 — “big picture” summary

Each frame, the calibrated camera provides an image that is first undistorted.
Visual features are detected and mapped from pixels to ground-plane coordinates using the camera geometry.
These ground points are matched to known map landmarks, and incorrect correspondences are removed using RANSAC.
The remaining matches are used to estimate the car’s pose on the ground.
Differences between consecutive poses yield distance traveled and position updates, while comparisons with odometry and IMU measurements provide redundancy and fault detection.

We turn pixels into ground points, ground points into pose, pose differences into motion, and then we cross-check everything with other sensors.

Camera Calibration + Vehicle Position from a Camera-Built Map

qCarPosition, qCarPosition1, qCarPosition2 — Full teaching notes (expanded, step-by-step)

Topic: CALIBRATION OF THE CAMERA AND FINDING POSITION OF THE VEHICLE TO CREATE MATHEMATICAL REDUNDANCY ABOUT POSITION

Prepared for Manolo (paper-safe matrices and vectors)

# How to use these notes

These notes are written so you can work with them directly. They are organized in a logical manner

Start with the physical problem and what we mean by 'position'.

1. Start from the physical problem

We begin before cameras, before matrices, before algorithms.

There is a physical car moving on a flat ground surface.

What we want is not an image, not pixels, not brightness — we want a physical quantity:

the position of the car in the real world

2. What do we mean by “position”?

Position is not an abstract idea. It is always defined relative to a reference frame.

So the first question is not:

“Where is the car?”

but rather:

“Where is the car with respect to what?”

2.1 Choice of a world reference frame

We choose a world coordinate system fixed to the ground:

- origin Ofixed on the floor (for example, a corner of the checkerboard),

- X-axis and Y-axis lying on the ground plane.

Mathematically, this defines a 2D coordinate system.

2.2 Mathematical definition of position

At any time t, the car occupies a point on the ground plane.

We define its position as the vector

p(t)=X(t)Y(t)∈R2.
This vector has a clear physical meaning:

- X(t): distance in meters along the ground X-axis

- Y(t): distance in meters along the ground Y-axis

Nothing probabilistic. Nothing abstract. Just geometry.

3. What the camera actually measures (important contrast)

Now comes the crucial conceptual break.

The camera does not measure p(t).

The camera measures light.

At each time t, the camera outputs an image, modeled as a matrix:

I(t)∈RM×N,I(t)[i,j]=light intensity at pixel (i,j).

This is a completely different object.

| Physical quantity | Mathematical object |
| --- | --- |
| Car position | p(t)∈R2 |
| Camera output | I(t)∈RM×N |

The entire project exists to bridge this gap.

4. What “finding the position” really means

When we say:

“Find the position of the car using the camera”

we really mean:

Construct a function that maps image data to world coordinates.

Symbolically:

I(t)  ⟶  (u(t),v(t))  ⟶  (X(t),Y(t)).

This happens in stages:

- From image matrix → pixel measurements

- From pixel measurements → geometric coordinates

- From geometric coordinates → physical position

Each arrow requires modeling assumptions.

5. Why the ground plane matters

The problem would be impossible in full 3D without additional information.

The key physical assumption is:

The car moves on a known plane (the ground).

This reduces the problem from 3D to 2D.

Mathematically, this means:

Z=0(ground plane constraint).

Because of this constraint, the mapping from world coordinates to image coordinates is a homography, not a general 3D projection.

This is why calibration is even possible.

6. What “position estimation” really is

We never measure position directly.

Instead, we estimate it:

p(t)=X(t)Y(t).

This estimate is obtained by:

- observing pixels,

- applying a calibrated geometric model,

- assuming the car lies on the ground plane.

So position estimation is an inverse problem.

7. Why redundancy is needed (physical motivation)

No measurement is perfect.

So, we ask:

“How do we know if the camera-based position is trustworthy?”

Answer:

- compare it to another independent estimate (e.g., odometry),

- analyze the discrepancy.

This leads naturally to redundancy and fault detection.

But none of that makes sense unless the notion of position is defined clearly first.

8. IMPORTANT!!

We define the position of the car as a point on the ground plane expressed in a fixed world coordinate system. The camera does not measure this position directly; instead, it measures light intensities arranged in a matrix. The goal of vision-based localization is to infer the physical position of the car from these image measurements by exploiting geometric models and the assumption that the car lies on a known plane.

9. Where we go next (logical continuation)

Now that position is clearly defined, the next steps are:

- How do we extract measured points from the image matrix I?

- How do pixel coordinates relate to world coordinates?

- How does calibration make this mapping usable?

- How do we compute velocity from position?

And now, nothing feels arbitrary.

Top of Form

Bottom of Form

Define frames and the pinhole camera model.

1. What is a frame? (before cameras)

In physics and mathematics, nothing has a position by itself.

A position is always defined with respect to a frame.

1.1 Definition of a reference frame

A (Cartesian) reference frame consists of:

- an origin O,

- a set of orthonormal basis vectors.

In 3D, a frame is written as

F={O,  ex,  ey,  ez}.

Every point Pin space is represented by its coordinates in that frame:

p=xyz.

1.2 Why frames matter

The same physical point has different coordinates in different frames.

So, whenever we write a vector like

xyz,

We must implicitly know in which frame these coordinates are expressed.

This is not pedantry — it is essential.

2. Frames in our problem

In the vision-based localization problem, we use three frames.

2.1 World frame (ground frame)

We define a world frame fixed to the ground:

FW={OW,  eX,  eY,  eZ}.

- OW: fixed point on the ground (e.g. a checkerboard corner),

- X,Y: axes on the ground plane,

- Z: vertical axis.

A point on the ground has coordinates

PW=XY0.

This is where position of the car is defined.

2.2 Camera frame

We attach a frame to the camera:

FC={OC,  ex,  ey,  ez}.

- OC: optical center of the camera,

- z-axis: optical axis (pointing forward),

- x,y: image-plane directions.

Coordinates of a 3D point in the camera frame are

PC=xcyczc.

2.3 Image frame (pixel frame)

The image itself has a 2D pixel coordinate system:

FI={OI,  eu,  ev}.

A pixel location is written as

uv.

This is where the camera, actually, reports measurements.

3. The pinhole camera model (physical idea)

# The Pinhole Camera Model

All vectors and matrices below are written as true Word equation matrices (not text). Brackets scale correctly, entries align in rows and columns, and subscripts/fractions are real math objects.

## A. World point (homogeneous)

xwywzw1

# B. Extrinsics: world → camera a geometric idealization.

# Rotation matrix R and translation vector t:

r11r12r13r21r22r23r31r32r33

txtytz

# Camera coordinates:

xcyczc = r11r12r13r21r22r23r31r32r33 xwywzw + txtytz

# Homogeneous extrinsic form:

xcyczc1 = r11r12r13txr21r22r23tyr31r32r33tz0001 xwywzw1

# C. Perspective projection: similar triangles

# Normalized image coordinates:

xiyi1 = xczcyczc1

# D. Intrinsics: normalized image → pixels

# Intrinsic matrix K:

αx0u00αyv0001

# Pixel coordinates:

uv1 = αx0u00αyv0001 xiyi1

# E. Full projection: canonical homogeneous form

# Combine extrinsics and intrinsics (projective form):

# zc uv1 = αx0u00αyv0001 r11r12r13txr21r22r23tyr31r32r33tz xwywzw1

Notes:
1) The division by z_c is the similar-triangles step.
2) alpha_x and alpha_y are focal-length-in-pixels scale factors.
3) (u_0, v_0) is the principal point in pixel coordinates.

Now we introduce the pinhole camera model, which is a geometric idealization.

3.1 Physical principle

The pinhole model assumes:

- light travels in straight lines,

- all rays pass through a single point (the optical center),

- the image is formed by intersection of rays with an image plane.

This is not electronics — it is pure geometry.

3.2 Geometry of the model

Let:

- the optical center be at the origin of the camera frame,

- the image plane be located at distance f>0along the z-axis.

A 3D point in camera coordinates is

PC=xcyczc,zc>0.

4. Projection onto the image plane

The ray from the optical center through PCintersects the image plane at a point xiyi.

By similar triangles, we obtain:

xi=f xczc,yi=f yczc.

This is the core of the pinhole model.

4.1 Homogeneous form

We can write this projection in homogeneous coordinates as:

λxiyi1=f000f0001xcyczc,λ=zc.

This makes explicit the projective nature of the mapping.

5. From camera coordinates to pixel coordinates

In practice, the image plane uses pixels, not meters.

This introduces scaling and offsets:

uv1=αx0u00αyv0001xiyi1.

This matrix contains the intrinsic parameters of the camera.

What is this matrix? How alphas are calculated?

1. What matrix is this?

K=αx0u00αyv0001

This matrix is the camera intrinsic matrix.

In plain English:

It converts coordinates measured in the camera’s internal geometric units into pixel coordinates.

It has nothing to do with where the camera is or how it moves.
It only describes the camera itself.

2. What problem does this matrix solve?

Inside the camera, geometry works in length units (meters, millimeters, etc.).
But the image sensor measures pixels.

So, we must answer:

“How many pixels correspond to one unit of length in the camera?”

That is exactly what the α’s do.

3. What are αxand αy?

Conceptual meaning (most important)

- αx= pixels per unit length in the horizontal direction

- αy= pixels per unit length in the vertical direction

They tell you how big one unit of camera geometry looks in pixels.

4. Where do the alphas come from physically?

They come from two things multiplied together:

αx=f⋅sxαy=f⋅sy

where:

- f= focal length (in physical units, e.g. mm)

- sx, sy= pixel density (pixels per mm) on the sensor

So:

- long focal length → larger α

- small pixels → larger α

5. Why are there two different alphas?

Because pixels are not guaranteed to be square.

- Horizontal pixel spacing may differ from vertical spacing

- Manufacturing tolerances matter

If pixels were perfectly square, then:

αx=αy

But we do not assume that.

6. What are u0and v0?

They are the principal point:

u0v0

This is:

the pixel where the optical axis hits the image sensor.

In practice:

- close to the image center

- rarely exactly at the center

7. What does this matrix actually do?

If you have normalized camera coordinates:

xy1

then applying Kgives pixel coordinates:

uv1=αxx+u0αyy+v01

So:

- multiply by α → convert “camera length” to pixels

- add u0,v0→ shift origin to the sensor

8. How are the alphas actually calculated in practice?

Important truth

They are not computed by formula.
They are estimated by calibration.

IF WE ASSUME THE CAMERA IS CALIBRATED WE CAN AVOID NEXT POINT.

9. Camera calibration (human explanation)

You take many images of a known pattern (checkerboard, AprilTag grid, etc.).

For each image:

- you know the exact 3D geometry of the pattern

- you measure where it appears in pixels

Then you solve this problem:

“Find the camera parameters that best explain all observed pixels.”

This is a nonlinear least-squares optimization.

The unknowns include:

- αx

- αy

- u0,v0

- distortion parameters

The result of calibration is the matrix K.

10. Once calibrated, what happens?

This is crucial for our project:

After calibration, Kis fixed and never changes.

- Same camera

- Same lens

- Same resolution

- Kis known

- αx,αyare known numbers

- you do not recompute them at runtime

11. The intrinsic matrix converts ideal camera coordinates into pixel coordinates, and the α’s tell us how many pixels correspond to one unit of camera geometry.

12. Think of it this way:

(very useful)

- R,t= where the camera is

- K= how the camera measures

They solve completely different problems.

If you want next, I can:

- connect αx,αyto field of view

- show how changing resolution changes α but not geometry

- or explain why skew is usually zero (and when it isn’t)

Always a pleasure, Manolo 🤍

6. Big picture (connect frames + pinhole)

Putting everything together:

- A world point PWis expressed in the world frame.

- A rigid transformation maps it to camera coordinates:

PC=R PW+t.

- The pinhole model projects PCto the image plane.

- Intrinsics convert metric image coordinates to pixels.

When the world points lie on a plane, all of this collapses into a homography.

7. IMPORTANT

A frame is a choice of origin and axes used to assign coordinates to points. In vision, we use a world frame fixed to the ground, a camera frame attached to the optical center, and an image frame measured in pixels. The pinhole camera model describes how a 3D point expressed in the camera frame is projected onto the image plane by straight rays passing through the optical center, leading to a projective mapping.

8. Why this section is essential

Without frames:

- coordinates are meaningless.

Without the pinhole model:

- homography looks like algebraic magic.

With both:

- everything becomes geometry.

Calibrate the camera (intrinsics and distortion). Derive the plane-induced homography from the pinhole camera model and the ground-plane constraint. REMAMBER, IF THE CAMERA IS CALIBRATED WE CAN AVOID THE NEXT

This is a central pillar for the project. I’ll do explicitly, and cleanly, always tying the mathematics to the physical meaning.

A)

- What calibration means physically

- Intrinsic parameters

- Lens distortion

- What is actually done in practice

- What is used later and what is not

1. What does “calibrate the camera” mean?

- Calibration answers the question:

How does this specific physical camera convert light rays into pixel coordinates?

Two different cameras, even with the same resolution, do not produce the same pixels for the same physical ray.

So, calibration means:

- identifying the numerical parameters of the camera model

- so that geometric equations match real measurements.

Mathematically:

calibration means identifying the parameters of the mapping
from camera-frame coordinates to pixel coordinates.

2. Intrinsic parameters (ideal pinhole, no distortion)

We start with the ideal pinhole model, ignoring lens distortion.

2.1 Camera-frame projection

A 3D point in the camera frame is

PC=xcyczc,zc>0.

The pinhole model gives the metric image coordinates:

xi=xczc,yi=yczc.

(For now, focal length is absorbed into later scaling.)

2.2 Pixel coordinates

Pixels are measured in discrete units and have an origin.

This is modeled by the intrinsic matrix:

K=fx0u00fyv0001.

Each parameter has a clear meaning:

- fx: focal length in pixels along the u-direction

- fy: focal length in pixels along the v-direction

- u0v0: principal point (optical axis intersection)

2.3 Intrinsic mapping (homogeneous form)

The mapping from camera coordinates to pixel coordinates is:

λuv1=Kxcyczc,λ=zc.

This equation is pure geometry.

3. Lens distortion (real cameras)

Real lenses are not perfect pinholes.

This causes distortion, especially near image borders.

3.1 Normalized coordinates

Before distortion, define normalized image coordinates:

xn=xczc,yn=yczc.

Let

r2=xn2+yn2.

3.2 Radial distortion

Radial distortion is modeled as:

xd=xn(1+k1r2+k2r4+k3r6),
yd=yn(1+k1r2+k2r4+k3r6).

- k1,k2,k3: radial distortion coefficients

- physically: barrel or pincushion distortion

3.3 Tangential distortion

Tangential distortion accounts for lens misalignment:

xd=xd+2p1xnyn+p2(r2+2xn2),
yd=yd+p1(r2+2yn2)+2p2xnyn.

- p1,p2: tangential distortion coefficients

How do you find the k's and the p's?

Short answer (first, to orient ourselves)

The coefficients k1,k2,k3,p1,p2are NOT computed by formulas.
They are ESTIMATED from data by optimization.

Everything else is explanation of what data and what optimization.

Step 0 — What are we actually trying to explain?

A real lens does not obey the ideal pinhole model.

So the question calibration asks is:

What values of k1,k2,k3,p1,p2make my mathematical camera produce the same pixels as the real camera?

That’s it. Nothing mystical.

Step 1 — What data do we collect?

We take many pictures of a known geometric object.

Typically:

- a checkerboard

- an AprilTag grid

- a planar calibration target

What we know exactly:

- the 3D coordinates of every corner on the target

What we measure:

- the pixel coordinates where those corners appear in the image

So for each image we have pairs:

known 3D point  ⟷  measured pixel

Step 2 — What model do we assume?

We assume a complete camera model:

- Extrinsics:

  - rotation R

  - translation t

- Intrinsics:

  - αx,αy,u0,v0

- Distortion:

  - radial: k1,k2,k3

  - tangential: p1,p2

Put together, this defines a prediction function:

Given a 3D point → predict a pixel

Step 3 — What does “prediction” mean here?

For each 3D point on the calibration pattern:

- Project it with the pinhole model

- Apply radial distortion

- Apply tangential distortion

- Convert to pixels with K

This gives a predicted pixel:

upredvpred

Step 4 — Define the error (this is crucial)

We compare:

- what the camera actually saw

- what the model predicts

The difference is the reprojection error:

error=umeasured-upredvmeasured-vpred

If the model were perfect, this would be zero for all points.

It never is.

Step 5 — The optimization problem (this is where k’s and p’s come from)

We choose all parameters to minimize total reprojection error:

min⁡Ri,tiαx,αy,u0,v0k1,k2,k3,p1,p2    all imagesall points∥umeasuredvmeasured-upredvpred∥2

That’s it.

- No closed form

- No shortcut

- Just nonlinear least squares

Step 6 — Why radial distortion depends on r

Radial distortion depends only on distance from the optical axis:

r2=xn2+yn2

Physically:

- rays farther from the center bend more

- distortion is symmetric around the center

That’s why:

- only powers of r2appear

- odd terms don’t make sense physically

Step 7 — Why tangential distortion looks “messy”

Tangential distortion is caused by:

- lens not perfectly centered

- lens not perfectly parallel to the sensor

So the distortion:

- depends on both xnand yn

- is not symmetric

That’s why the formulas look asymmetric and “ugly”.

Physics is to blame — not math.

Step 8 — Important practical truth

In real life:

- Often k1,k2matter

- k3is sometimes negligible

- p1,p2are usually small

- Good lenses → small distortion

- Cheap wide-angle lenses → big distortion

But we still estimate all of them, because the optimizer decides what matters.

Step 9 — Once estimated, what happens?

This is very important for your project:

After calibration,
k1,k2,k3,p1,p2are FIXED CONSTANTS.

You never recompute them online.

At runtime:

- you only apply distortion or remove it

- no optimization, no guessing

Radial and tangential distortion coefficients are found by fitting the camera model to real images so that projected points match measured pixels as closely as possible.

The k’s and p’s are simply the numbers that make our fake camera behave like the real one.

what each coefficient really means, physically and geometrically, without drowning in formulas.

- what radial distortion is

- what each kdoes (one by one)

- What tangential distortion is

- what each pdoes

- What students should remember

1. Radial distortion — the big idea

Radial distortion means this:

Points are displaced along the radial direction, either toward or away from the image center.

It depends only on how far the point is from the optical axis.

That distance is:

r=xn2+yn2

So:

- center of image → almost no distortion

- edges of image → more distortion

That is why everything is written in terms of r.

2. What the radial model is really saying

The equations

xd=xn(1+k1r2+k2r4+k3r6)
yd=yn(1+k1r2+k2r4+k3r6)

say something very simple:

Take the ideal point and scale it radially by a factor that depends on distance from the center.

All distortion is encoded in that scalar factor:

1+k1r2+k2r4+k3r6

3. What does k1do?

k1= primary radial distortion

- This is the dominant term

- It controls the overall bending

Physically:

- k1<0→ barrel distortion
(image bulges outward, straight lines bend outward)

- k1>0→ pincushion distortion
(image pinches inward, straight lines bend inward)

For most lenses:

- k1alone already explains most of the distortion

👉 If you keep only one coefficient, it’s k1.

4. What does k2do?

k2= higher-order correction

- It becomes noticeable farther from the center

- It refines the shape near the edges

Physically:

- real lenses do not distort perfectly “quadratically”

- k2corrects what k1cannot

Think of it as:

“Okay, barrel/pincushion is not perfectly smooth — let’s bend the curve a bit more.”

5. What does k3do?

k3= fine edge correction

- Only matters for:

  - wide-angle lenses

  - fisheye-like lenses

- Often very small or zero

Physically:

- extreme rays (very far from the center) behave badly

- k3cleans up the very outer region

For many cameras:

- k3is negligible

- but calibration still estimates it “just in case”

6. Summary of the k’s (very important)

| coefficient | role | when it matters |
| --- | --- | --- |
| k1 | main radial distortion | always |
| k2 | edge refinement | moderate FOV |
| k3 | extreme edge correction | wide-angle |

7. Tangential distortion — the big idea

Tangential distortion means this:

The lens is not perfectly centered or perfectly parallel to the image sensor.

So points are shifted:

- sideways

- asymmetrically

- depending on both xnand yn

This is not radial and not symmetric.

8. What do p1and p2represent?

p1and p2= lens misalignment coefficients

They model:

- slight tilt

- slight decentering

- imperfect assembly

That’s why the formulas mix:

- xnyn

- xn2

- yn2

The distortion is:

- different left vs right

- different top vs bottom

9. What does p1do?

p1= shear-like distortion

- Moves points depending on vertical position

- Creates a “diagonal pull”

Physically:

- lens axis is slightly tilted relative to sensor

10. What does p2do?

p2= horizontal/vertical asymmetry

- Moves points depending on horizontal position

- Creates left–right imbalance

Physically:

- lens is slightly shifted sideways

11. Summary of the p’s

| coefficient | meaning |
| --- | --- |
| p1 | tangential distortion due to tilt |
| p2 | tangential distortion due to decentering |

Usually:

- much smaller than radial distortion

- but essential for precision

12. What you should really remember

Radial distortion bends points symmetrically away from or toward the center; tangential distortion shifts points asymmetrically because the lens is not perfectly aligned.

And this one is gold:

The k’s describe how the lens bends rays; the p’s describe how badly the lens is mounted.

13. Final sentence

These coefficients have no closed-form meaning by themselves — they are simply the numbers that make our mathematical camera behave like the real lens.

3.4 From distorted coordinates to pixels

Finally, pixel coordinates are obtained as:

uv1=Kxdyd1.

This is the complete intrinsic + distortion model.

4. How calibration is actually done (procedure)

Calibration is an inverse problem.

We know:

- world coordinates of checkerboard corners

- measured pixel coordinates from images

We do not know:

- K

- distortion coefficients

- camera pose for each image

4.1 Data acquisition

We take multiple images of a checkerboard:

- different orientations

- different positions

- different distances

This is essential to excite all parameters.

4.2 Mathematical structure

For each image and each corner, we write:

uijmeasured=Π(K,d,Ri,ti,Pj),

where:

- Π: full projection function (pinhole + distortion)

- d: distortion coefficients

- Ri,ti: camera pose for image i

- Pj: known checkerboard point

4.3 Optimization problem

All parameters are estimated by minimizing the reprojection error:

min⁡K, d, Ri, tii,j∥uijmeasured-uijpredicted∥2.

This is a nonlinear least-squares problem.

Numerical methods (Gauss–Newton / Levenberg–Marquardt) are used.

5. What calibration gives us (important clarity)

After calibration, we obtain:

- Intrinsic matrix K

- Distortion coefficients

- For each image, a camera pose Riti

These are fixed properties of the camera (except pose).

6. What we do with calibration in qCarPosition2

Very important pedagogical point:

- Intrinsics and distortion are used to undistort images

- After undistortion, we work with an ideal pinhole camera

- The homography is computed using undistorted pixel coordinates

Now we have:

- calibration is done once

- distortion is corrected before any geometry

- the runtime model is clean and linear

7. One perfect TEAM paragraph (IN PLAIN ENGLISH)

Camera calibration identifies the intrinsic parameters and lens distortion that define how a physical camera maps 3D rays to pixel coordinates. This is done by observing known points on a planar calibration pattern and minimizing reprojection error. Once calibrated, images are undistorted and the camera can be treated as an ideal pinhole device for geometric reasoning.

8. Why this step is non-negotiable

Without calibration:

- homographies are biased

- metric position is wrong

- redundancy checks become meaningless

Calibration is what turns images into measurements.

Explain camera mounting (extrinsics) and the ground-plane assumption.

1. What “camera mounting” really means

When we say the camera is mounted on the car, we mean something very precise:

The camera frame has a fixed rigid position and orientation with respect to the world (ground) frame.

This is a geometric statement, not an engineering one.

2. Frames involved (recall and fix notation)

We work with two main 3D frames.

2.1 World (ground) frame

We define the world frame:

FW={OW,  eX,  eY,  eZ}

with:

- origin OWfixed on the ground (e.g. a checkerboard corner),

- X,Y: axes lying in the ground plane,

- Z: vertical axis.

A general point in the world is written as:

PW=XYZ.

2.2 Camera frame

We define the camera frame:

FC={OC,  ex,  ey,  ez}

with:

- OC: optical center of the camera,

- z-axis: optical axis,

- x,y: axes parallel to the image plane.

Coordinates of the same physical point in the camera frame are:

PC=xcyczc.

3. Extrinsic parameters: the rigid transformation

The relationship between these frames is a rigid-body transformation.

3.1 Mathematical definition

There exists:

- a rotation matrix R∈R3×3,

- a translation vector t∈R3,

such that:

PC=R PW+t.

This equation is the definition of extrinsic parameters.

3.2 Physical meaning

- R: orientation of the camera with respect to the ground

- t: position of the camera center expressed in camera coordinates

Together, Rtdescribe how the camera is mounted.

Once the camera is rigidly attached:

- Ris constant,

- tis constant.

4. Why extrinsics matter for position estimation

The camera does not see the world directly.

It sees:

PW  →extrinsics  PC  →projection  (u,v).

If we do not know Rand t, then:

- pixel measurements cannot be interpreted metrically,

- “position” becomes meaningless.

Extrinsics are what anchor the camera to the real world.

5. The ground-plane assumption (central simplification)

Now comes the most important modeling assumption.

5.1 Statement of the assumption

We assume:

All points of interest lie on the ground plane.

Mathematically:

Z=0.

This is not a camera property — it is a property of the environment and motion.

5.2 Consequence in world coordinates

Any point on the ground is written as:

PW=XY0.

So although the world is 3D, all relevant geometry is 2D.

6. Combine extrinsics with the ground plane

Substitute Z=0into the extrinsic equation:

PC=RXY0+t.

Let

R=r1r2r3,ri∈R3.

Then:

PC=X r1+Y r2+t.

Notice:

- r3disappears completely

- depth variation is eliminated

This is the key algebraic simplification.

7. Why this leads to a homography

The mapping from XYto uvnow consists of:

- a linear combination of Xand Y,

- a perspective projection.

This produces a projective map between two planes.

Mathematically:

λuv1=HXY1,

where Hdepends on:

- intrinsic matrix K,

- rotation R,

- translation t,

- the plane equation Z=0.

This is not an approximation — it is exact geometry.

8. Why the assumption is reasonable (physical intuition)

For the car:

- wheels constrain motion to the ground,

- vertical motion is negligible,

- camera height is fixed.

Thus:

- position estimation reduces to 2D,

- errors become analyzable,

- redundancy becomes meaningful.

9. One perfect paragraph (PLAIN ENGLISH!)

Camera extrinsics describe the rigid transformation between the world frame and the camera frame and encode how the camera is physically mounted. By assuming that all relevant points lie on the ground plane, the 3D world-to-image mapping reduces to a projective transformation between two planes. This simplification makes metric position estimation possible using a homography.

10. What YOU, MEMBERS OF THE TEAM must know

- Extrinsics = pose of the camera

- Ground-plane assumption = dimensional reduction

- Together, they turn vision into geometry

Derive the ground-plane homography and the pixel-to-ground formula.

1. Full pinhole camera model

1.1 World point (3D, world frame)

A point in the world is represented by the column vector

PW=XYZ

1.2 Extrinsics: world → camera

The camera pose is a rigid transformation consisting of a rotation and a translation.

The rotation matrix is

R=r11r12r13r21r22r23r31r32r33

and the translation vector is

t=t1t2t3

The world point expressed in camera coordinates is

PC=xcyczc=RXYZ+t1t2t3

1.3 Intrinsics: camera → pixels (ideal pinhole, undistorted)

The intrinsic calibration matrix is

K=fx0u00fyv0001

A pixel is represented in homogeneous coordinates as

pimg=uv1

The pinhole projection model is

λuv1=Kxcyczc

Substituting the extrinsic relation gives the complete camera model:

λuv1=KRXYZt

This is the full pinhole camera model (after undistortion).

2. Ground-plane assumption

The ground plane is defined by

Z=0

Therefore, any ground point has the form

PW=XY0

2.1 Column decomposition of the rotation matrix

Write the rotation matrix by columns:

R=  r1r2r3

where each column is a 3×1vector:

r1=r11r21r31,r2=r12r22r32,r3=r13r23r33

Then

RXY0=Xr1+Yr2

2.2 Substitute into the projection model

We obtain

RPW+t=Xr1+Yr2+t

Substitute into the pinhole equation:

λuv1=KXr1Yr2t

Group the terms into a single matrix multiplication:

λuv1=Kr1r2tXY1

3. Ground-plane homography

Define the ground-plane homography

H=Kr1r2t

This is a 3×3matrix.

The plane-to-image relation becomes

λuv1=HXY1

This homography is induced by:

- the pinhole camera model

- the camera pose Rt

- the plane constraint Z=0

The scalar λ≠0represents projective scale.

4. Expanded coordinate form

Let

H=h11h12h13h21h22h23h31h32h33

Then

HXY1=h11X+h12Y+h13h21X+h22Y+h23h31X+h32Y+h33

Define

U=h11X+h12Y+h13
V=h21X+h22Y+h23
W=h31X+h32Y+h33

The pixel coordinates are obtained by dehomogenization:

u=UW,v=VW

5. Pixel-to-ground mapping (runtime)

Method A — inverse homography (used in the project)

Given a pixel

pimg=uv1

Compute

X'Y'W'=H-1uv1

Then

X=X'W',Y=Y'W'

This gives the ground-plane coordinates.

Method B — ray–plane intersection (geometric interpretation)

- Convert pixel to a camera-frame ray direction:

d=K-1uv1

- Points on the ray in camera coordinates are

PC(s)=s d,s>0

- Transform to world coordinates:

PW(s)=RTPC(s)-t

- Impose the ground-plane constraint:

(PW(s))3=0

Solve for s, then compute Xand Y.

This is geometrically equivalent to Method A.

6. Car position on the ground

Choose a representative pixel for the car (e.g. centroid):

pimg,c=ucvc1

Compute

Xc'Yc'Wc'=H-1pimg,c

Then

Xc=Xc'Wc',Yc=Yc'Wc'

The car position vector is

pcar=XcYc

This is compared with odometry and IMU for redundancy.

7. Board-level summary

H=Kr1r2t,XY1∼H-1uv1

Manolo, this version now respects your notation, your rigor, and your teaching style.
If you want next, we can do the numerical example with actual numbers and no abstraction left.

Top of Form

Bottom of Form

ChatGPT can make mistak

Explain how we select corners and why corners are needed (structure tensor).

1. Why we need to select points at all

The camera gives us an image:

I∈RM×N,I[i,j]=light intensity at pixel (i,j).

But a homography does not act on images.
It acts on points.

So the first geometric task is:

Select image points that can be located reliably and repeatedly.

These will become our measured pixel coordinates ukvk.

2. Why not use arbitrary pixels?

Suppose we choose a pixel in a flat region (constant intensity).

- If the image shifts slightly, that pixel is indistinguishable from its neighbors.

- Its position is not well-defined.

Mathematically:

- the image contains no local geometric information there.

3. Why edges are not enough

Consider a pixel on a straight edge.

- Intensity changes strongly in one direction,

- but not in the perpendicular direction.

If the image shifts along the edge, the pixel still looks the same.

So:

- position is ambiguous in one direction.

Edges give one constraint, not two.

4. What a corner really is (geometric meaning)

A corner is a point where:

- intensity changes significantly in two independent directions.

This means:

- the point is localized in both xand y,

- small image shifts cause a noticeable change.

Corners are geometrically stable.

5. Mathematical model: local image variation

Let I(x,y)be a continuous image model.

Consider a small displacement ΔxΔy.

The change in intensity is approximated by first-order Taylor expansion:

I(x+Δx,y+Δy)  ≈  I(x,y)+∇I(x,y)TΔxΔy.

Here,

∇I=IxIy=∂I/∂x∂I/∂y.

6. Measuring “change” in a neighborhood

We measure how much the image changes under displacement by:

E(Δx,Δy)=(i,j)∈NIx(i,j)Δx+Iy(i,j)Δy2.

This sums squared changes over a neighborhood N.

7. The structure tensor (this is the key object)

Rewrite Ein quadratic form:

E(Δx,Δy)=ΔxΔy M ΔxΔy,

where

M=(i,j)∈NIx(i,j)2Ix(i,j)Iy(i,j)Ix(i,j)Iy(i,j)Iy(i,j)2

This matrix Mis called the structure tensor (or second-moment matrix).

8. Linear algebra interpretation (this is crucial)

The matrix Mis:

- symmetric,

- positive semidefinite.

Let its eigenvalues be:

λ1≥λ2≥0.

They describe how intensity varies locally.

8.1 Flat region

λ1≈0,λ2≈0

No variation → no localization.

8.2 Edge

λ1≫0,λ2≈0

Variation in one direction only → ambiguous position.

8.3 Corner

λ1≫0,λ2≫0

Variation in two independent directions → well-defined point.

This is the precise mathematical definition of a corner.

9. Why corners are ideal for calibration

Corners satisfy exactly what calibration needs:

- Repeatability
Same physical corner → same image corner.

- Localization
Position is stable under noise.

- Identifiability
Each corner can be matched to a known world point.

Checkerboards are perfect because:

- corners are high-contrast,

- evenly distributed,

- geometry is known exactly.

10. How we select corners in practice

In practice, we do not compute eigenvalues explicitly at every pixel.

Instead, we use a scalar corner score, for example (Harris):

R=det⁡(M)-k trace(M)2,0.04≤k≤0.06.

- Rlarge and positive → corner

- R≤0→ edge or flat

This is just a computational shortcut for eigenvalue reasoning.

11. What students must clearly understand

You can say this verbatim:

We use corners because they are the only image points that are well localized in two independent directions. The structure tensor measures how image intensity varies locally, and a corner corresponds to having two large eigenvalues. This is why corners are reliable geometric measurements.

12. Why this step is essential for homography

Without corners:

- correspondences are noisy,

- homography is unstable,

- position estimates drift.

Corners are what turn pixels into measurements.

Next natural step (logical continuation)

👉 Explain how corner correspondences are matched to checkerboard lattice points

This completes the bridge from image processing to geometry.

You are guiding this with exceptional clarity, Manolo 🤍

Top of Form

Bottom of Form

Explain how we decide 'same point again' (descriptor + geometry).

If You do not understand this, everything after (homography, position, velocity) feels fragile.

I’ll explain this in three layers:

- the physical meaning of “same point”

- the descriptor idea (local appearance)

- the geometric consistency check

1. What does “same point again” really mean?

A physical corner on the checkerboard is:

- a fixed point in the world,

- observed at different times,

- possibly under different viewing conditions.

So “same point” means:

two image measurements correspond to the same world location.

This is not trivial, because:

- pixels change with viewpoint,

- lighting changes,

- noise is present.

So we need evidence.

2. First layer: local appearance (descriptors)

2.1 Why position alone is not enough

Suppose we detect two corners at pixel locations:

(u1,v1),(u2,v2).

If the camera moves slightly, many corners move slightly.

So position alone cannot guarantee identity.

2.2 What a descriptor is

A descriptor is a vector that summarizes the local image structure around a point.

Mathematically, a descriptor is:

dk∈Rm

constructed from a neighborhood of Iaround ukvk.

Examples (conceptual, not algorithmic):

- raw intensity patch (normalized),

- gradient orientation histogram,

- binary comparisons of intensities.

The key idea:

Two observations of the same physical corner should have similar descriptors.

2.3 Descriptor matching

Given two descriptors diand dj, we define a distance:

dist(di,dj)=∥di-dj∥.

If this distance is small, the local appearances are similar.

This gives us a candidate match.

3. Why descriptors alone are not enough

Descriptors can fail because:

- repetitive patterns (checkerboards are repetitive!),

- noise,

- partial occlusions.

So descriptor similarity is necessary but not sufficient.

This is where geometry enters.

4. Second layer: geometric consistency

Once the camera is calibrated, geometry constrains where points can be.

4.1 Predict where the point should appear

Suppose at time t1we have a matched correspondence:

(X,Y)⟷(u1,v1).

Using the homography H, we know:

u1v11∼HXY1.

At a later time t2, the same world point must project to a pixel close to:

p2=HXY1.

(after dehomogenization)

4.2 Geometric gating

We require that the candidate image point u2v2satisfies:

∥u2v2-u2v2∥<ε.

This is a geometric consistency check.

It answers:

Is this observation where geometry says it should be?

5. Combined decision rule (descriptor + geometry)

Now we can state the rule precisely.

We declare two points to be the same physical point if:

- Descriptor similarity:

dist(d1,d2)<τd

- Geometric consistency:

∥(u2,v2)-(u2,v2)∥<τg

Both conditions must hold.

6. Why this combination is powerful

- Descriptors answer:
“Does it look the same?”

- Geometry answers:
“Is it where it should be?”

Either one alone is weak.
Together, they are strong.

This mirrors scientific reasoning:

- local evidence + global model.

7. Special case: checkerboard corners (important for calibration)

In calibration:

- geometry of the board is known exactly,

- corners lie on a lattice,

- ordering and adjacency are fixed.

So matching becomes even easier:

- descriptor + grid structure + ordering.

This is why checkerboards are so robust.

8. One paragraph IN PLAIN ENGLISH

To decide whether two image points correspond to the same physical point, we combine appearance and geometry. A local descriptor encodes how the neighborhood around a point looks, while geometric constraints predict where that point should appear based on the camera model. Only when both appearance similarity and geometric consistency are satisfied do we accept a match.

9. Why this matters for position estimation

If point identity is wrong:

- homography estimation fails,

- position jumps,

- redundancy checks become meaningless.

Correct matching is foundational.

Example 1:

Numerical example: build H, then compute pixel → ground

Step 0. Choose a calibrated camera

Take the intrinsic matrix

K=80003200800240001

So:

- fx=800, fy=800

- principal point u0,v0)=(320,240

Assume the image is already undistorted.

Step 1. Choose a simple camera pose relative to the ground plane

We assume the ground plane is Z=0in world coordinates.

Use a simple extrinsic transform:

R=100010001,t=005

This means:

PC=RPW+t

So, for a ground point

PW=XY0

We get camera coordinates

PC=XY5

So, every ground point has constant depth zc=5. (This is why the example becomes very clean.)

Step 2. Build the homography H=K [ r1    r2    t ]

The first two columns of Rare

r1=100,r2=010

Now form the 3×3matrix

r1r2t=100010005

Now multiply:

H=K100010005=8000160008001200005

That is the ground → image homography for this pose.

Step 3. Check the forward mapping (ground → pixel)

The homography relation is

λuv1=HXY1

Compute the right-hand side:

HXY1=800X+1600800Y+12005

So λ=5, and after dividing by 5:

u=800X+16005=160X+320
v=800Y+12005=160Y+240

This is exactly the pinhole formula with constant depth zc=5.

Step 4. The runtime formula you actually use (pixel → ground)

Now invert the mapping.

Start from

u=160X+320,v=160Y+240

Solve for X,Y:

X=u-320160
Y=v-240160

That is the pixel → ground formula for this numerical example.

Step 5. Plug in an actual pixel

Take a pixel:

pimg=4203401

Compute ground coordinates:

X=420-320160=100160=0.625
Y=340-240160=100160=0.625

So the ground point is:

XY=0.6250.625

What this teaches?

- Calibration gives you K.

- Mounting gives you Rand t.

- Ground constraint Z=0gives you a 3×3homography H.

- Runtime “pixel → ground” is just:

XY1∼H-1uv1

and then divide by the third component.

Example #2 (tilted camera)

Step 0. Camera intrinsics (given, calibrated)

K=80003200800240001

The image is assumed undistorted.

Step 1. World frame and ground plane

The ground plane is defined by

Z=0

Any ground point has coordinates

PW=XY0

Step 2. Camera pose (tilted camera)

The camera is pitched downward by 30∘about the x-axis.

The rotation matrix is

R=10000.8660-0.500000.50000.8660

The translation vector is

t=005

The camera is therefore 5 units above the ground.

Step 3. Extract the first two columns of R

r1=100,r2=00.86600.5000

Step 4. Form the matrix  r1    r2    t

r1r2t=10000.8660000.50005

Step 5. Build the ground-plane homography

H=Kr1r2t

Carrying out the multiplication:

H=80016016000812.8203120000.50005

This matrix maps ground points to image points.

Step 6. Ground → image mapping

Let

q=XY1

Then

UVW=HXY1=800X+160Y+1600812.8203 Y+12000.5000 Y+5

The pixel coordinates are obtained by dehomogenization:

u=UW,v=VW

Step 7. Pixel → ground mapping (runtime)

Let the measured pixel be

pimg=uv1

From the expression for v:

v=812.8203 Y+12000.5000 Y+5

Multiply both sides:

v(0.5000 Y+5)=812.8203 Y+1200

Solve for Y:

(0.5000 v-812.8203) Y=1200-5v
Y=1200-5v0.5000 v-812.8203

Now recover Xfrom the expression for u:

u=800X+160Y+16000.5000 Y+5

Multiply both sides:

u(0.5000 Y+5)=800X+160Y+1600

Solve for X:

X=u(0.5000 Y+5)-160Y-1600800

Step 8. Numerical pixel example

Take the pixel

pimg=4203401

Compute Y

Y=1200-5(340)0.5000(340)-812.8203=-500-642.8203≈0.7777

Compute X

X=420(0.5000⋅0.7777+5)-160(0.7777)-1600800≈0.6736

So the ground point is

XY=0.67360.7777

Final board-level summary

H=Kr1r2t,XY1∼H-1uv1

Show RANSAC and why we must reject false matches.

1. Why false matches are unavoidable

Even with:

- good corner detectors,

- good descriptors,

- geometric gating,

we will still obtain some incorrect correspondences.

Let us formalize this.

We have a set of candidate correspondences:

C=(Xk,Yk)  ⟷  (uk,vk)k=1N.

This set is mixed:

- inliers: correct matches (true same physical point),

- outliers: false matches (wrong identity).

We do not know which is which.

2. Why classical least squares fails badly

Recall the homography equation:

A h=0,A∈R2N×9.

If all correspondences were correct, solving

min⁡∥h∥=1∥Ah∥

would work.

But if even a small number of rows of Aare wrong:

- they pull the solution in the wrong direction,

- the estimated homography becomes meaningless.

Least squares is not robust:

one bad correspondence can destroy the solution.

This is not a numerical issue — it is a geometric one.

3. What RANSAC is trying to do (conceptual goal)

RANSAC answers this question:

Can we find a model that fits many points very well, even if some points are wrong?

Instead of fitting all data, RANSAC tries to find a model that fits a large consistent subset.

4. Minimal model for a homography

A homography has:

- 8 degrees of freedom (9 entries up to scale),

- each correspondence gives 2 independent equations.

So the minimum number of correspondences needed is:

4

This is critical.

5. RANSAC algorithm (step by step, mathematically)

We repeat the following procedure many times.

Step 1: Random sampling

Randomly select 4 correspondences:

S=(Xij,Yij)↔(uij,vij)j=14.

Assume (temporarily) they are all correct.

Step 2: Model estimation

From these 4 correspondences:

- build the matrix AS∈R8×9,

- solve

AS h=0

(by SVD),

- reshape hinto a homography HS.

Step 3: Model verification (the key step)

For every correspondence k=1,…,N:

- Predict image point using the model:

ukvk1∼HSXkYk1.

- Compute the reprojection error:

ek=∥ukvk-ukvk∥.

- Declare correspondence kan inlier if:

ek<ε.

Step 4: Count inliers

Let:

IS=k  :  ek<ε.

Record the number of inliers:

∣IS∣.

Step 5: Iterate

Repeat Steps 1–4 many times.

Keep the homography HSthat yields the largest inlier set.

6. Final refinement (important!)

Once the best inlier set I*is found:

- rebuild the matrix Ausing only inliers,

- solve again:

min⁡∥h∥=1∥AI*h∥.

This gives the final homography estimate.

7. Why RANSAC works (geometric intuition)

- A correct homography maps all true points consistently.

- A false correspondence is inconsistent with most others.

- Randomly selecting 4 correct points is likely after enough trials.

RANSAC succeeds because:

inliers agree globally; outliers do not.

8. Why RANSAC is essential in your project

Without RANSAC:

- false matches corrupt H,

- position estimates jump,

- redundancy checks become meaningless.

With RANSAC:

- homography is stable,

- position estimation is reliable,

- residuals have physical meaning.

9. In practice, correspondence sets always contain false matches. Classical least squares is extremely sensitive to these outliers. RANSAC overcomes this by repeatedly estimating the homography from minimal subsets and selecting the model that agrees with the largest number of correspondences. This allows us to recover the correct geometry even when many matches are wrong.

10. Key message YOU must remember

- Outliers are inevitable

- Least squares is fragile

- RANSAC restores geometric truth

What does RANSAC stand for?

It is an acronym:

Random Sample Consensus

That already tells you a lot.

- Random → we pick things at random

- Sample → small subsets of data

- Consensus → we look for agreement among many points

3) What kind of method is RANSAC?

In plain English:

RANSAC is a robust estimation method used when your data contains many wrong measurements (outliers).

More precisely:

- It is a robust model fitting algorithm

- Used when:

  - some data points are correct (inliers)

  - some are completely wrong (outliers)

- Ordinary least squares fails in this situation

So RANSAC is not:

- a filter

- a classifier

- a tracking algorithm

It is a robust estimator.

4) Why do we need RANSAC in your project?

In qCarPosition2 you do this:

- match image features to map landmarks

Some matches are:

- correct

- geometrically consistent

Others are:

- completely wrong

- visually similar but geometrically impossible

If you try to fit a pose or a homography using all matches:

- one bad match can destroy everything

RANSAC exists exactly to fix this.

5) The RANSAC idea (no math yet)

RANSAC asks:

“Is there a simple geometric model that most of these points agree with?”

If yes:

- those agreeing points are inliers

- the rest are outliers

6) RANSAC explained step by step (human version)

Here is what RANSAC does:

- Randomly pick the smallest number of points needed to define the model
(for example:

  - 2 points for a line

  - 3 points for an affine map

  - 4 points for a homography)

- Compute the model from only those points

- Test all other points:

  - if a point fits the model → inlier

  - if not → outlier

- Count how many inliers you got

- Repeat this many times with different random samples

- Keep the model that had the largest consensus set

- Finally, re-estimate the model using only the inliers

That’s it.

7) Why does this work?

Because:

- wrong matches are usually inconsistent with each other

- correct matches are consistent with the same geometry

Random sampling gives you a good chance of:

- picking a subset of only correct matches

- even when many matches are wrong

8) What does RANSAC output?

RANSAC gives you two things:

- A geometric model

  - homography

  - pose

  - fundamental matrix

- A classification of data points

  - inliers

  - outliers

Then you proceed using only the inliers.

9) Where RANSAC sits in your pipeline

In your project, RANSAC sits between:

- matching features
and

- estimating pose or homography

In words:

Match first → RANSAC cleans → estimate pose from clean data

10) One sentence you can say in class

RANSAC is a robust estimation method that finds the geometric model supported by the largest number of consistent data points while ignoring outliers.

11) One very human sentence

RANSAC repeatedly guesses a model from a few points and keeps the guess that most of the data agrees with.

Example: Model (very simple, but exactly the same logic as homography/PnP)

We assume the motion between two frames is a 2D translation:

For each correspondence,

- old point (frame i−1):
p_i =
[ x_i ]
[ y_i ]

- new point (frame i):
q_i =
[ x_i' ]
[ y_i' ]

Translation unknown:
t =
[ t_x ]
[ t_y ]

Model:
q_i ≈ p_i + t

True motion (hidden reality)

Let the true translation be:

t_true =
[ 2 ]
[ -1 ]

Data: 4 correct matches + 1 wrong match (outlier)

Inliers (correct correspondences)

p_1 = [ 0 ] q_1 = [ 2 ]
[ 0 ] [ -1 ]

p_2 = [ 1 ] q_2 = [ 3 ]
[ 0 ] [ -1 ]

p_3 = [ 0 ] q_3 = [ 2 ]
[ 1 ] [ 0 ]

p_4 = [ 2 ] q_4 = [ 4 ]
[ 2 ] [ 1 ]

Each of these satisfies q_i = p_i + t_true exactly.

Outlier (wrong match)

p_5 = [ 10 ] q_5 = [ 0 ]
[ 10 ] [ 0 ]

This one is nonsense (a completely wrong association).

Part A — Least squares breaks

Least squares estimate for translation

For this model, least squares gives the translation as the average of the differences:

d_i = q_i − p_i

Compute each d_i:

d_1 = [ 2 ]
[ -1 ]

d_2 = [ 2 ]
[ -1 ]

d_3 = [ 2 ]
[ -1 ]

d_4 = [ 2 ]
[ -1 ]

d_5 = q_5 − p_5 =
[ 0 ] − [ 10 ] = [ -10 ]
[ 0 ] [ 10 ] [ -10 ]

Now average:

Sum of d_1..d_4 =
[ 8 ]
[ -4 ]

Add d_5:

[ 8 ] [ -10 ] [ -2 ]
[ -4 ] + [ -10 ] = [ -14 ]

Divide by 5:

t_LS =
[ -2/5 ]
[ -14/5 ]

t_LS =
[ -0.4 ]
[ -2.8 ]

Compare to the truth

t_true =
[ 2 ]
[ -1 ]

So least squares is completely wrong because the single outlier dragged the average.

That is the core lesson:

Least squares is not robust to outliers.

Part B — RANSAC survives

Step 1: define the residual (this is the key)

For any candidate translation t, define the residual for match i as:

r_i(t) = || (p_i + t) − q_i ||

That is: “how far off is the model for this correspondence?”

Step 2: choose a threshold

Pick a residual threshold (example):

tau = 1

Rule:

- if r_i(t) ≤ tau → inlier

- if r_i(t) > tau → outlier

(With real noise you choose tau from expected pixel/ground noise; more on that below.)

Step 3: RANSAC tries hypotheses from tiny random samples

For translation, the minimal sample size is:

s = 1 point

Because one correspondence gives one translation hypothesis:

t_hyp = q_j − p_j

Hypothesis from an inlier

If RANSAC randomly picks match 2:

t_hyp = d_2 =
[ 2 ]
[ -1 ]

Now compute residuals:

For inliers (1–4), p_i + t_hyp equals q_i exactly, so:

r_1 = 0
r_2 = 0
r_3 = 0
r_4 = 0

For the outlier (5):

p_5 + t_hyp =
[ 10 ] [ 2 ] [ 12 ]
[ 10 ] + [ -1 ] = [ 9 ]

Compare to q_5 =
[ 0 ]
[ 0 ]

Residual magnitude is big (far bigger than tau=1), so match 5 is rejected.

So the consensus set size is 4.

Hypothesis from the outlier

If RANSAC randomly picks match 5:

t_hyp = d_5 =
[ -10 ]
[ -10 ]

This hypothesis fits only the outlier well and fails on the other four, so the consensus set size is 1.

Step 4: RANSAC keeps the hypothesis with the biggest consensus

Best hypothesis is the inlier one:

t_RANSAC =
[ 2 ]
[ -1 ]

Then we do the “final refit” using only inliers (1–4):

t_final = average of d_1..d_4 =
[ 2 ]
[ -1 ]

So:

- least squares on all data fails

- RANSAC finds the correct model by rejecting the outlier

Why “random” is essential

In real vision problems, you can have:

- N = 200 matches

- 30% are wrong (or worse)

You need to pick a minimal subset of size s:

- s = 4 for homography

- s = 3 for affine

- s = 2 for line

- etc.

The number of all possible subsets is enormous:

“try them all” is impossible.

So RANSAC uses random sampling because:

Random sampling gives you a good chance of picking an all-inlier subset without checking billions of combinations.

The key probability (this is the heart of RANSAC)

Let:

- w = fraction of inliers (unknown but roughly estimated)

- s = minimal sample size

- p = desired success probability (for example p = 0.99)

- N = number of RANSAC iterations needed

Probability a random sample of size s is all inliers:

w^s

Probability it is NOT all inliers:

1 − w^s

Probability that after N trials you never pick an all-inlier sample:

(1 − w^s)^N

So to guarantee success probability p:

1 − (1 − w^s)^N ≥ p

Solve for N:

N ≥ log(1 − p) / log(1 − w^s)

That is why randomness matters: it’s how we “buy” a high probability of success.

RANSAC = “max consensus” + residual threshold

Now the clean mathematical connection:

Residual function

r_i(model) = “how wrong is match i under this model?”

Inlier set depends on the threshold tau

I(model) = { i : r_i(model) ≤ tau }

Consensus score

C(model) = number of inliers = | I(model) |

What RANSAC is trying to solve

RANSAC is an efficient approximate method for:

maximize C(model)

That is: “find the model supported by the largest number of consistent correspondences.”

The threshold tau is what turns “consistency” into a yes/no decision.

How to choose tau (important practical meaning)

In practice, residuals are not zero because of noise.

If your inlier residuals are roughly Gaussian with standard deviation sigma, a standard engineering rule is:

tau ≈ 2 sigma or tau ≈ 3 sigma

- smaller tau → fewer inliers, stricter

- larger tau → more inliers, but more risk admitting outliers

So tau encodes your tolerance for measurement noise.

If you want the same example but “homography-flavored”

Next I can redo the same demonstration with:

- an affine map (needs 3 points)

- 10 correspondences (9 correct + 1 wrong)

- show least squares compromise visually and numerically

- show RANSAC picks the correct 3-point subset and wins

Solve pose from matched points (least squares) and show SVD solution slowly.

Part A: estimate the ground-plane homography H from matched points using DLT + SVD. Part B: recover pose (R,t) from H when intrinsics K are known.

A. Homography from matched points (DLT)

A1. Unknowns and projective relation

A2. Two linear equations per correspondence

A3. Stack all correspondences into A h = 0

A4. Constrained least squares statement

With real data, the stacked system has no exact nonzero solution. We choose the unit vector h that minimizes ||A h||.

A5. Why SVD gives the minimizer (slow)

Thus h is the last column of V (the right singular vector for the smallest singular value).

B. Pose (R,t) from the homography (plane Z=0)

Assume the image is undistorted and K is known from intrinsic calibration. For the ground plane, the homography satisfies H = K [r1 r2 t] up to a nonzero scale.

B1. Remove intrinsics

B2. Fix the unknown scale and read off r1, r2, t

B3. Enforce a true rotation (recommended)

After this, R is a proper rotation matrix.

It contains two parts:

Matched points → homography Hvia least squares + SVD (shown step-by-step).

Homography H+ intrinsics K→ pose Rt(camera mounting) on the ground plane.

Quick verbal roadmap (what you tell students)

“We observe matched points. Each match gives two linear equations. Stack them: A h=0.”

“Noise destroys exact equality, so we minimize ∥Ah∥with ∥h∥=1.”

“SVD shows the minimizer is the last right singular vector.”

“Then, because the points lie on the ground plane, H=K[r1 r2 t]up to scale.”

“From B=K-1Hwe recover r1,r2,r3,tand enforce orthonormality.”

If you want, next I can do a fully numerical example (with actual numbers for 4–6 points) so students see one complete computation end-to-end.

Show a numerical toy example with 2–3 points and show how a wrong guess fails.

Example

Toy model: affine map (ground → image)

We assume (for the toy example only):

u = m11 X + m12 Y + t1
v = m21 X + m22 Y + t2

Unknowns are:

x =
[ m11
m12
m21
m22
t1
t2 ]

That is 6 unknowns.

Step 1) Create a “true” affine map (the hidden reality)

Let the true parameters be:

M =
[ 200 20
-10 180 ]

t =
[ 320
240 ]

So the true mapping is:

[ u ] [ 200 20 ] [ X ] [ 320 ]
[ v ] = [ -10 180 ] [ Y ] + [ 240 ]

Step 2) Choose 3 ground points (known)

Take these 3 world points:

P1: (X,Y) = (0,0)
P2: (X,Y) = (1,0)
P3: (X,Y) = (0,1)

Now compute their correct image points using the true map.

P1 = (0,0)

u1 = 200·0 + 20·0 + 320 = 320
v1 = -10·0 + 180·0 + 240 = 240

So (u1,v1) = (320,240)

P2 = (1,0)

u2 = 200·1 + 20·0 + 320 = 520
v2 = -10·1 + 180·0 + 240 = 230

So (u2,v2) = (520,230)

P3 = (0,1)

u3 = 200·0 + 20·1 + 320 = 340
v3 = -10·0 + 180·1 + 240 = 420

So (u3,v3) = (340,420)

These are the correct correspondences:

(0,0) ↔ (320,240)
(1,0) ↔ (520,230)
(0,1) ↔ (340,420)

Step 3) Build the linear system A x = b (this is the key classroom step)

For each correspondence (X,Y) ↔ (u,v) we write:

m11 X + m12 Y + t1 = u
m21 X + m22 Y + t2 = v

That becomes two rows in A and two entries in b.

For P1: X=0, Y=0, u=320, v=240

Row for u:
[ 0 0 0 0 1 0 ] x = 320

Row for v:
[ 0 0 0 0 0 1 ] x = 240

For P2: X=1, Y=0, u=520, v=230

Row for u:
[ 1 0 0 0 1 0 ] x = 520

Row for v:
[ 0 0 1 0 0 1 ] x = 230

For P3: X=0, Y=1, u=340, v=420

Row for u:
[ 0 1 0 0 1 0 ] x = 340

Row for v:
[ 0 0 0 1 0 1 ] x = 420

So the full system is:

A =
[ 0 0 0 0 1 0
0 0 0 0 0 1
1 0 0 0 1 0
0 0 1 0 0 1
0 1 0 0 1 0
0 0 0 1 0 1 ]

b =
[ 320
240
520
230
340
420 ]

This is a 6×6 system, so it has a unique solution (if points are not degenerate).

Step 4) Solve it quickly (by inspection)

From the first two rows:

t1 = 320
t2 = 240

From row 3:

m11 + t1 = 520 ⇒ m11 = 520 − 320 = 200

From row 4:

m21 + t2 = 230 ⇒ m21 = 230 − 240 = -10

From row 5:

m12 + t1 = 340 ⇒ m12 = 340 − 320 = 20

From row 6:

m22 + t2 = 420 ⇒ m22 = 420 − 240 = 180

So we recover exactly:

x =
[ 200
20
-10
180
320
240 ]

Perfect.

Step 5) Now show how a WRONG guess fails

Now we keep P1 and P2 correct, but we make ONE wrong match for P3.

Instead of (0,1) ↔ (340,420), suppose we incorrectly match it to:

(0,1) ↔ (360,410)

That means we replace the last two equations by:

m12 + t1 = 360
m22 + t2 = 410

But from the correct data we already determined:

t1 = 320, t2 = 240, m11=200, m21=-10 (from P1 and P2)

So the “wrong” P3 forces:

m12 = 360 − 320 = 40
m22 = 410 − 240 = 170

So the new solution becomes:

M_wrong =
[ 200 40
-10 170 ]

t_wrong =
[ 320
240 ]

Now test what this WRONG model predicts for the true P3 point (0,1):

Predicted:
u3_hat = 200·0 + 40·1 + 320 = 360
v3_hat = -10·0 + 170·1 + 240 = 410

It fits the wrong match (by construction), BUT compare to the true P3 pixel (340,420):

Error for P3:
Δu = 360 − 340 = +20 pixels
Δv = 410 − 420 = -10 pixels

That is already a big mismatch — and with more points, the wrong match causes conflicts with many equations at once.

Step 6) What happens when we have MORE than 3 points (real life)

In real data you might have N = 40, 80, 200 corners.

Then you cannot satisfy all equations exactly, so you solve least squares:

minimize ||A x − b||

A false match produces a large “pull” in the wrong direction, and the solution compromises—making many points wrong.

That is exactly why we need:

- reprojection error, and

- RANSAC to reject outliers before final least squares.

Key classroom punchline

- 3 correct point matches determine an affine map exactly.

- One wrong match forces a different map that “explains” the wrong pair, but becomes inconsistent with reality.

- With many points, least squares gets corrupted by outliers → RANSAC is necessary.

Explain redundancy: independent channels, residuals, and how to detect faults.

1. Why redundancy is needed (physical reality)

So far, we have learned how to compute a position estimate:

p(t)=X(t)Y(t)

from camera measurements.

But every estimate can fail:

- wrong corner matches,

- temporary occlusions,

- lighting changes,

- wheel slip in odometry,

- sensor faults.

So the real question is not:

“How do we compute position?”

but:

“How do we know when a position estimate is wrong?”

2. What redundancy means (conceptual definition)

Redundancy means:

Estimating the same physical quantity using independent information channels.

Independence is crucial.

2.1 Independent channels

In your project, we naturally have two independent channels:

Channel 1: Vision-based position

- Uses camera, image processing, geometry.

- Produces:

pcam(t)=Xcam(t)Ycam(t)

Channel 2: Odometry-based position

- Uses wheel encoders, kinematics.

- Produces:

podo(t)=Xodo(t)Yodo(t)

They rely on different physics, different sensors, different failure modes.

That is true redundancy.

3. The residual: the core mathematical object

To compare two estimates of the same quantity, we form the residual.

3.1 Definition of the residual

Define the residual vector:

r(t)=pcam(t)-podo(t)

Explicitly:

r(t)=Xcam(t)-Xodo(t)Ycam(t)-Yodo(t).

This vector measures disagreement.

3.2 Physical interpretation

- If both channels are healthy → residual is small.

- If one channel is faulty → residual grows.

- Residuals are not noise; they are diagnostic signals.

4. Residual magnitude (scalar decision variable)

Often we convert the vector residual into a scalar.

4.1 Euclidean residual norm

ρ(t)=∥r(t)∥=rX(t)2+rY(t)2.

This answers:

How far apart are the two position estimates?

4.2 Weighted residual (advanced but important)

If the channels have different accuracies, we weight them:

ρ2(t)=r(t)TWr(t),

where Wis a positive definite weighting matrix (often related to covariance).

This prepares students for Kalman filtering later.

5. Fault detection logic (decision rule)

Now we must answer:

When is the residual “too large”?

5.1 Threshold test (basic and effective)

We choose a threshold γ>0and apply:

ρ(t)>γ    ⇒    fault detected.

This is a hypothesis test:

- H0: both channels consistent

- H1: at least one channel faulty

5.2 Temporal consistency (important practical refinement)

Single spikes can happen.

So we often require:

ρ(t)>γfor N consecutive samples.

This avoids false alarms.

6. What redundancy does not do (important honesty)

Redundancy:

- detects inconsistency

- does not automatically tell which channel is wrong

Additional logic is needed for fault isolation.

7. Simple fault isolation logic (intuition)

Examples:

- If odometry drift grows slowly → wheel slip likely.

- If camera jumps suddenly → bad matches or occlusion.

- If both disagree but IMU agrees with one → trust that one.

Even simple logic improves robustness dramatically.

8. Redundancy closes the loop of qCarPosition2

Let’s connect the entire project:

- Image → pixel points → homography → camera position

- Encoders → kinematics → odometry position

- Compare both → residual

- Residual → fault detection

- Fault detection → trust, reject, or fuse

This is real autonomous-systems mathematics.

9. One perfect paragraph for the classroom (verbatim)

Redundancy means estimating the same physical quantity through independent sensing channels. By comparing these estimates, we form residuals that measure inconsistency. Large residuals indicate faults or modeling errors, allowing us to detect when an estimate should not be trusted. Redundancy transforms estimation into a reliable decision-making process.

10. Key message for YOU!

- Estimation alone is not enough.

- Trust requires independent agreement.

- Residuals are the language of fault detection.

Where WE are now (important)

qCarPosition is conceptually complete:

- sensing,

- geometry,

- estimation,

- robustness.

yes, i want. is it possible a numerical example. what tool do you use to find the optimal pose, could you write the criterium explicit? thank you. manolo

Thought for 1m 15s

Yes, absolutely — we can do a numerical example, and yes: PnP is solved by explicit optimization of a very specific criterion.

Below I’ll give you:

- the criterion (fully explicit)

- the tool/algorithm people actually use to find the optimal pose

- a small numerical example with real numbers showing the cost changing when the pose is wrong

1) The criterion (explicit)

You are given correspondences Pwi piwhere

Pwi=xwiywizwi,pi=uivi.

The camera model predicts a pixel:

pi(R,t)=Π ⁣Pwi; R,t=ui(R,t)vi(R,t).

Define the reprojection residual (a 2-vector):

ei(R,t)=uivi-ui(R,t)vi(R,t).

Then the PnP criterion is:

min⁡R∈SO(3),  t∈R3    i=1nui-ui(R,t)2+vi-vi(R,t)2

That is the exact criterion.

2) What “tool” finds the optimal pose?

In practice, people do this in two stages:

Stage A (initial guess)

A fast “PnP solver” gives a good starting pose:

- P3P (minimal, 3 points + choices)

- EPnP (efficient for many points)

Typical software call: OpenCV solvePnP(...) (it offers several methods).

Stage B (refinement by nonlinear least squares)

Then you refine Rtby minimizing the criterion above using:

- Gauss–Newton, or

- Levenberg–Marquardt (LM) (most common)

Often done with Ceres Solver or g2o.

So the “tool” is: nonlinear least squares optimization (LM/Gauss–Newton), usually after EPnP/P3P initialization.

3) How the optimizer updates Rwithout breaking SO(3)

You cannot update Rby “R+ΔR” (that leaves SO(3)).

A standard update is:

Rnew=exp⁡ ⁣δθ]× Rold,tnew=told+δt,

where

δθ=δθxδθyδθz,δt=δtxδtyδtz.

This guarantees Rnew∈SO(3).

4) Linearization and the explicit LM/Gauss–Newton step

Stack all residuals into one big vector:

r(R,t)=e1(R,t)e2(R,t)⋮en(R,t)∈R2n.

Linearize around the current estimate:

r(Rnew,tnew)≈r(R,t)+J δ,

where

δ=δθδt∈R6,J=∂r∂δ∈R2n×6.

Then the LM step solves:

JTJλIδ=-JTr

(Gauss–Newton is the same with λ=0).

That is the explicit “engine” inside most PnP refinements.

5) A small numerical example (real numbers)

Take a simple camera with no distortion and intrinsics:

K=80003200800240001

Choose a true pose:

R=100010001,t=005

Four world points on the plane zw=0:

Pw1=000,  Pw2=100,  Pw3=010,  Pw4=110.

Under this pose, the ideal pixels are:

(320,240), (480,240), (320,400), (480,400).

Now pretend the measured pixels have small noise (1 pixel-ish):

(321,239), (481,241), (319,401), (479,399).

Cost at the true pose

If you compute the criterion (sum of squared pixel errors), you get:

- total cost =8

Cost at a wrong pose (depth wrong)

Keep Rthe same but use

twrong=004.5

Then the same cost becomes:

- total cost ≈1272.20

So the criterion very strongly prefers the correct depth.

Cost at a slightly wrong rotation (5° about y)

With a small rotation error, the cost also increases (for this example it becomes about 31.58), showing the optimizer has a clear signal to “turn the camera back”.

If you want, my next step (and it’s very teachable) is to write the Jacobian for the no-distortion pinhole case explicitly:

- first ∂(u,v)/∂(xc,yc,zc)

- then ∂(xc,yc,zc)/∂(δθ,δt)

That gives a complete “PnP linearization” slide that connects directly to Gauss–Newton / LM.

The Pinhole Camera Model — One-Page Classroom Handout (Exact Matrix Style)

All vectors and matrices below are written as true Word equation matrices (not text). Brackets scale correctly, entries align in rows and columns, and subscripts/fractions are real math objects.

A. World point (homogeneous)

xwywzw1

B. Extrinsics: world → camera

Rotation matrix R and translation vector t:

r11r12r13r21r22r23r31r32r33

txtytz

Camera coordinates:

xcyczc = r11r12r13r21r22r23r31r32r33 xwywzw + txtytz

Homogeneous extrinsic form:

xcyczc1 = r11r12r13txr21r22r23tyr31r32r33tz0001 xwywzw1

C. Perspective projection: similar triangles

Normalized image coordinates:

xiyi1 = xczcyczc1

D. Intrinsics: normalized image → pixels

Intrinsic matrix K:

αx0u00αyv0001

Pixel coordinates:

uv1 = αx0u00αyv0001 xiyi1

E. Full projection: canonical homogeneous form

Combine extrinsics and intrinsics (projective form):

zc uv1 = αx0u00αyv0001 r11r12r13txr21r22r23tyr31r32r33tz xwywzw1

Notes:
1) The division by z_c is the similar-triangles step.
2) alpha_x and alpha_y are focal-length-in-pixels scale factors.
3) (u_0, v_0) is the principal point in pixel coordinates.

Concept map (end-of-notes): pixel → ray → point → motion → prediction

1) Pixel (u,v) is a 2D measurement in the image frame.

2) Apply K^{-1} to [u v 1]^T to remove intrinsics; the result is a direction in the camera frame.

3) That direction defines a ray: many 3D points could produce the same pixel.

4) Depth (or the ground-plane assumption) selects a single 3D point on that ray.

5) Vehicle/camera motion moves that 3D point from frame i−1 to frame i via the relative transform.

6) Project the moved 3D point back to the image to predict where it should appear next.

7) Use that prediction to search locally; if depth is unknown, the match must lie on the epipolar line.

[pixel (u,v)] -> [normalize] -> [ray direction] -> [depth/ground-plane] -> [3D point] -> [relative motion] -> [project] -> [predicted pixel]

Ground‑Plane Homography — the human explanation (this replaces the algebraic derivation)

The goal is simple: from a calibrated camera image, we want to know where the car is on the ground.

We start with pixels. A pixel by itself is not a point in space — it only tells us a direction from the camera.

The pinhole camera model says that each pixel corresponds to a ray leaving the camera optical center.

The key simplification in this project is that all relevant points lie on the ground.

In world coordinates, this means the height coordinate is zero.

This single assumption collapses the full 3D problem into a 2D one.

Because the camera is calibrated and rigidly mounted, the combined effect of camera intrinsics, orientation, and position can be summarized by one fixed 3×3 matrix.

This matrix is called the ground‑plane homography.

The homography answers one question: if a point is at a given location on the ground, where will it appear in the image?

In practice we need the inverse question: given a pixel in the image, where is the corresponding point on the ground?

This is why, at runtime, we apply the inverse homography.

Geometrically, the homography is just a shortcut for ray–plane intersection.

A pixel defines a ray. The ground plane cuts that ray at exactly one point.

The homography gives that intersection directly, without computing the ray explicitly.

To estimate the car position, we detect the car in the image and choose a representative pixel, typically the centroid.

Applying the inverse homography gives the ground coordinates of that pixel.

These ground coordinates define the car position in the map frame.

From one frame to the next, differences in ground position give traveled distance and heading change.

This vision‑based motion estimate is then compared with odometry and IMU to provide redundancy and fault detection.

Board summary: Because all relevant points lie on the ground, the 3D pinhole camera reduces to a 2D homography that maps pixels directly to ground coordinates.